تلخيص دورة ccna شبكة

Static route

For switch to connect to route

int f0/1

no shutdown

ip address 13.0.0.1 255.0.0.0

exit

for first network in another router to see second network

the Configuration in the second route

ip route 20.200.200.0 255.255.255.0 10.0.0.2

rip route

this on all route have network(switch)

route rip

version 2

network 10.0.0.0

network 11.0.0.0

eigrp

route eigrp 1

version 2

network 10.0.0.0

network 11.0.0.0

network 192.168.3.0

do wr

do show ip route

ospf

route ospf 1

network 192.168.3.0 0.0.0.255 area 0

network 10.0.0.0 0.255.255.255 area 0

network 11.0.0.0 0.255.255.255 area 0

route ospf 1

route-id 90.0.0.1

int loopback 1

ip address 50.0.0.1 255.0.0.0

when you cut any cable the second router will send masseg

i am activ

hsrp

int e1/1

standby 5 ip 192.168.1.50

access control list

stander

en

config t

access-list 1 deny 192.168.2.3

access-list 1 permit any

int e1/0

ip access-group 1 out

extended

منع 30.0.0.3 من الوصول الى 20.0.0.2

access-list 100 deny tcp host 30.0.0.3 host 20.0.0.2 eq 80

access-list 100 permit ip any any

int f0/0

ip access-group 100 in

do wr

do show ac

tcp eq 23

tcp eq 25

udp eq 53

icmp echo # for ping

nat

static

route ospf 1

network 14.0.0.0 0.255.255.255 area 0

network 12.0.0.0 0.255.255.255 area 0

do show ip route

ip route 0.0.0.0 0.0.0.0 f0/0

ip nat inside source static 192.168.2.2 10.0.0.2

dynamic

en

config t

route ospf 1

network 11.0.0.0 0.255.255.255 area 0

network 12.0.0.0 0.255.255.255 area 0

network 10.0.0.0 0.255.255.255 area 0

do wr

do show ip route

access-list 1 permit 192.168.2.0 0.0.0.255

ip nat pool aa 10.0.0.2 10.0.0.4 netmask 255.255.255.0

ip nat inside source list 1 pool aa

int e1/0

ip nat inside

int f0/0

ip nat outside

backup route

do copy run tftp

192.168.1.2

Osam

do copy tftp run

192.168.1.2

Osam

do show flash

do copy flash tftp

securing route

securing console

line console

password 11

login

line console 0

no password

username osam password aa

username ali password aa

line console 0

loging local

username test secret aa

privilege mod

enable password or secret aa

telnet & ssh

enable secret aa

line vty 0 4

password aa

login

do wr

vlan

vlan 2

name car

int f0/1 or int range f0/1-2

switchport access vlan 2

exit

the access is trunk between to switch

int f0/24

switchport mod trunk

system in vlan 2 can see system in another switch in vlan 2

vlan management

AFTER Create vlan

enable secret 123

login

int vlan 2

ip address 192.168.1.100 255.255.255.0

line vty 04

password 123

native

switchport trunk allowed vlan 4